SUPPLEMENTAL MATERIALS

TARGET-RESPONSE-BLIND PORTABILITY AUDIT OF SPARSE AXIAL-LOAD MONITORING LAYOUTS FOR PILES UNDER LIQUEFACTION-INDUCED DOWNDRAG

Complete notation, secondary checks and supporting result tables. The main manuscript defines the primary claims; these materials retain all secondary evidence and assumptions.

Notation

The following symbols are used in the main text and these materials:

| Symbol | Definition |
| --- | --- |
| CI0.95 | descriptive 95% earthquake-cluster bootstrap interval; |
| Cb | complete set of admissible layouts for budget b; |
| D | candidate set of measured station locations; |
| DS,b* | layout selected on source test S for budget b; |
| Epe(D) | pile–event 90th-percentile NRMSE for layout D; |
| G | set of common measured gauge levels; |
| I[·] | indicator function; |
| PT(DS,b*) | fraction of admissible target layouts outperformed by the frozen source-selected layout; |
| Qpet(zi) | measured axial load at normalised depth zi; |
| Q̂pet(zi¦D) | axial load reconstructed from layout D; |
| Qpetmax | maximum measured axial load at sampled time t; |
| Q(f)(zi) | published or digitally interpreted axial load at level i in independent profile f; |
| S | source physical test; |
| Tpe | deterministically sampled times for a pile–event history; |
| Ub | uniform discrete comparator for budget b; |
| b | number of additional stations beyond the fixed upper boundary; |
| e | earthquake-event index; |
| f | index of an external axial-load reference profile; |
| i, j, k | measured-level indices; |
| kpet⋆ | index of the first measured-grid level attaining the maximum axial load; |
| m | number of measured reference levels; |
| m(f) | number of published or digital reference levels in profile f; |
| ns | total retained station count, ns = b + 1; |
| p | pile index; |
| qα{·} | empirical quantile at probability α; |
| rpet(D) | snapshot NRMSE; |
| r(f)(D) | static-profile NRMSE for layout D in independent profile f; |
| s0 | NRMSE floor, 0.05 MN in the reported specification; |
| spet | snapshot-specific normalisation scale; |
| t | sampled-time index; |
| xi | axial coordinate of measured level i; |
| xu, xl | upper and lower coordinates of the common measured span; |
| zi | normalised coordinate over the measured span; |
| zpetmax | normalised depth of the discrete peak-load level; |
| Δe(D) | earthquake-level median paired candidate-minus-uniform difference; |
| εQ,pet(D) | relative reconstructed peak-load magnitude error for layout D; |
| εz,pet(D) | absolute reconstructed peak-depth error on the normalised measured grid; |
| κpet,i | normalised absolute second difference at internal measured level i; |
| ẼT(D) | median target-event error of layout D; |
| A | reference levels below the candidate layout’s deepest retained station; |
| ei(D) | normalised reconstruction residual at reference level i for layout D; |

Table S1. Robustness and secondary checks

| Check | Comparison | Result | Interpretation |
| --- | --- | --- | --- |
| 12 specifications | SKS02 → SKS03, b = 2–4 | Lower error in 12/12 specifications at each budget | Direction and layout invariant |
| 12 specifications | SKS03 → SKS02, b = 3–4 | Lower error in 12/12 specifications at each budget | Direction and layout invariant |
| 12 specifications | SKS03 → SKS02, b = 2 | +0.0230 to +0.0236 | Higher-error direction persists |
| Partial six-level grid | SKS02 layout → SKS03 Pile 1, b = 2 | Median −0.180; better in 5/5 | Secondary, no target imputation |

Source: Sinha et al. (2020a, b). Candidate-minus-uniform differences are paired within earthquake; added budget b excludes the fixed upper station.

Table S2. Objective-specific transfer and high-budget saturation

| Selection → evaluation | Added b | Median Δ NRMSE | Median Δ peak load | Median Δ peak depth | Events better: load/depth |
| --- | --- | --- | --- | --- | --- |
| SKS02 → SKS03 | 2 | −0.116 | −0.108 | +0.000 | 5/5; 1/5 |
| SKS02 → SKS03 | 3 | −0.055 | +0.013 | +0.071 | 1/5; 0/5 |
| SKS02 → SKS03 | 4 | −0.086 | +0.014 | +0.071 | 0/5; 0/5 |
| SKS02 → SKS03 | 5 | −0.0005 | +0.016 | +0.071 | 1/5; 1/5 |
| SKS03 → SKS02 | 2 | +0.023 | −0.112 | +0.214 | 5/6; 0/6 |
| SKS03 → SKS02 | 3 | −0.006 | +0.030 | +0.143 | 1/6; 0/6 |
| SKS03 → SKS02 | 4 | −0.053 | −0.319 | −0.357 | 6/6; 6/6 |
| SKS03 → SKS02 | 5 | +0.0000 | +0.000 | +0.000 | 0/6; 0/6 |

Source: Sinha et al. (2020a, b). Candidate-minus-uniform differences are paired within earthquake; added budget b excludes the fixed upper station.

Digital field-reference protocol

A further digital check uses five Delft screw-injection piles: F1, F2, T1, T2 and T3 (Duffy et al. 2024, 2025). The 470-mm screw-tip diameter and pile lengths of 19.2–20.8 m accompany a layered deposit with a deep sand bearing layer. Fundex casings were removed; Tubex casings remained. F3 is excluded following source-reported casing-extraction problems. The source converts distributed fibre-optic strain to force using its stiffness interpretation; these processed curves are not a new strain-to-force calibration.

Delft profiles are taken from the last complete record within each numbered loading stage, excluding 45 s at each stage boundary. The 53 selected profiles are evaluated on 0.20-m reference grids, with 0.40 m as a sensitivity, because the instrument resolution is 0.20 m although source processing interpolates at 0.01 m. The separate head-load coordinate and recorded gaps are retained. Exact source indices divided by seven define the frozen requested depths; nearest-level mapping and Eq. (11) are unchanged. Paired stage differences are summarised by their median within each pile, then equally across five piles. Original and boundary-constrained layouts, linear/PCHIP reconstruction and distance-weighted error are all retained; no external result selects a protocol.

The coverage decomposition follows Eq. (14) in the main manuscript. It separates where reconstruction error occurs without identifying its mechanical cause.

The 53 stages comprise 12 each for F1 and F2, 15 for T1, eight for T2 and six for T3. Source file hashes, selected timestamps and complete-stage eligibility are supplied. F3 is excluded by source quality, not by reconstruction performance. All comparisons use frozen source layouts. The 0.01-m source grid is interpolated and does not represent independent observations at that spacing.

Table S3. Delft pile-level median paired NRMSE differences: primary linear, original-layout, 0.20-m-grid protocol

| Source | Added b | F1 | F2 | T1 | T2 | T3 |
| --- | --- | --- | --- | --- | --- | --- |
| SKS02 | 2 | +0.143 | +0.154 | +0.111 | +0.197 | +0.164 |
| SKS02 | 3 | +0.018 | +0.003 | +0.035 | +0.031 | +0.018 |
| SKS02 | 4 | +0.002 | -0.014 | -0.005 | +0.005 | +0.015 |
| SKS03 | 2 | +0.018 | +0.005 | +0.021 | +0.083 | +0.031 |
| SKS03 | 3 | +0.002 | -0.002 | +0.052 | +0.024 | +0.006 |
| SKS03 | 4 | -0.013 | -0.015 | -0.028 | +0.003 | +0.007 |

Source: Duffy et al. (2025), with frozen layouts from Sinha et al. (2020a, b). Negative differences favour the candidate; ±0.005 is the stated equivalence tolerance. Five piles belong to one site; stages are repeated measurements. All other protocols and distance-weighted results are supplied as CSV files.

Table S4. Graphical-resolution stress scenarios for 24 published-profile comparisons

| Pixels | Draws per comparison | Classes unchanged in all draws | Median of comparison-wise minima |
| --- | --- | --- | --- |
| 1 | 2,000 | 23/24 | +0.021 |
| 2 | 2,000 | 19/24 | +0.020 |
| 4 | 2,000 | 17/24 | +0.017 |

Source: digitised profiles from Kevan (2017) and FHWA (2017). Amplitudes are specified graphical-resolution scenarios, not confidence bounds. Turrell load bounds also include half the between-figure range; its published gauge depths remain fixed. Smooth FHWA perturbations use eleven depth anchors shared by candidate and comparator. Full scenario outputs include changed mappings and minimum/maximum paired errors.

Table S5. Independent published-profile consistency relative to equal-budget uniform placement

| Evidence | Profiles | n | L / E / H | Median Δr(f) | Inference role |
| --- | --- | --- | --- | --- | --- |
| Primary marked field profiles | Turrell H and pipe | 12 | 1 / 0 / 11 | +0.030 | Primary external field check |
| Published field traces | Christchurch 12-m and 14-m CFA | 12 | 0 / 2 / 10 | +0.022 | Secondary trace-level check |
| Quality-qualified field sensitivity | Turrell concrete | 6 | 3 / 1 / 2 | −0.016 | Sensitivity only |
| Resolution-limited centrifuge sensitivity | Cambridge MS09 | 4 | 0 / 1 / 3 | +0.020 | Sensitivity only |

Source: Data from Kevan (2017), FHWA (2017), and Stringer (2012).

Note: Δr(f) denotes frozen candidate minus uniform NRMSE; negative values favour the candidate. L/E/H denotes lower, practically equivalent, and higher error using a ±0.005 tolerance. Counts are descriptive profile comparisons, not independent field trials or probability estimates.

Table S6. Sampling, comparator-population and protocol-sensitivity summary

| Check | Direction or evidence | b or protocol | Result | Interpretive boundary |
| --- | --- | --- | --- | --- |
| Leave-one-event-out | SKS02 source | b = 2/3/4 | Exact match 100/100/100%; better 5/6, 6/6, 6/6 | Source-sample stability only |
| Leave-one-event-out | SKS03 source | b = 2/3/4 | Exact match 80/60/80%; better 5/5 at each b | Source-sample stability only |
| All admissible layouts | SKS02 → SKS03 | b = 2/3/4 | Outperformed 71/63/83%; ranks 6/21, 13/35, 6/35 | Target responses used only for audit |
| All admissible layouts | SKS03 → SKS02 | b = 2/3/4 | Outperformed 43/69/86%; ranks 12/21, 11/35, 5/35 | Target responses used only for audit |
| Lower boundary retained | SKS02 → SKS03 | b = 2 | Candidate 0.113; uniform 0.279; median Δ = −0.170 | Configuration sensitivity |
| Lower boundary retained | SKS03 → SKS02 | b = 2 | Candidate 0.210; uniform 0.218; median Δ = −0.009 | Primary higher-error result is protocol dependent |
| Full-scale profiles | Original layout | Linear / PCHIP | L/E/H = 1/2/21 and 2/5/17; median Δ = +0.022/+0.024 | Descriptive, tolerance ±0.005 |
| Full-scale profiles | Boundary-constrained | Linear / PCHIP | L/E/H = 1/2/21 and 2/7/15; median Δ = +0.025/+0.012 | Descriptive, tolerance ±0.005 |

Source: Data from Sinha et al. (2020a, b), Kevan (2017), and FHWA (2017).

Note: b is the number of stations added to the fixed upper boundary; ns = b + 1. L/E/H denotes lower, practically equivalent and higher error. Ranks use every admissible target-grid layout and do not alter the frozen layout.

Phase-specific diagnostic protocol

Input acceleration is in seconds; axial-load and Keller pore-pressure times are converted from minutes. Their timestamps match exactly for all 22 eight-level histories. At each fast/slow join, only times exceeding every preceding timestamp are retained, removing one pressure row per SKS02 event and two per SKS03 event; axial records receive the same rule. Missing responses are not interpolated. The whole-history benchmark uses file order; this diagnostic uses elapsed time.

Shaking spans 5–95% of the cumulative trapezoidal integral of squared input acceleration. The pressure proxy is the 90th percentile across available non-flat Keller channels, clipping negative ratios to zero, then applying a centred 3-s elapsed-time mean. At least 80% of channels must be available; missing values are not imputed. The reference peak spans shaking onset to 60 s after shaking ends. Decay begins at the first post-shaking timestamp starting an observed interval of at least 60 s entirely below the chosen peak fraction, with no consecutive gap exceeding 60 s.

The primary peak fraction is 0.50; 0.25 and 0.75 are sensitivities. A reference peak below 0.10 precludes decay partitioning, excluding SKS02 EQM1 from post-shaking phases but not shaking. These operational thresholds do not define liquefaction or complete dissipation. Before-decay and after-decay periods extend from shaking end to the boundary and from the boundary to the common record end (Table S7).

Loads are unsmoothed or use centred elapsed-time means of 6 s for Pile 1 and 3 s otherwise, with half-window phase-edge guards. Up to 120 distinct measured times nearest evenly spaced elapsed-time targets are retained per phase; 240 is a sensitivity. At least ten snapshots are required. Three peak fractions, two smoothing choices and two caps give 12 settings. Repeated shaking results across fractions are not independent checks. Elapsed-time smoothing avoids treating slow-logging row windows as few-second intervals.

Original and previously boundary-retained layouts remain fixed for b = 2–4. Equations (3), (4), (9) and (10) define snapshot errors; Eq. (5) is restricted to each phase. Equation (7) aggregates paired pile differences within earthquakes. Event medians, ranges and signs are descriptive, without new population intervals. Table S8 and Fig. S4 show b = 2; CSV files retain every budget, response objective and setting.

Table S7. Phase boundaries for the primary decay fraction of 0.50

| Test / event | Shaking (s) | Pressure peak | Decay time (s) | Preceding interval (s) |
| --- | --- | --- | --- | --- |
| SKS02 E1 | 1.21–14.98 | 0.017 | Not assigned | — |
| SKS02 E2 | 2.72–13.39 | 0.681 | 692.38 | 0.008 |
| SKS02 E3 | 2.02–12.11 | 0.999 | 552.21 | 0.008 |
| SKS02 E4 | 2.53–13.19 | 0.439 | 1961.62 | 39.883 |
| SKS02 E5 | 4.51–14.94 | 0.965 | 681.87 | 39.961 |
| SKS02 E6 | 3.52–56.27 | 0.966 | 292.65 | 0.008 |
| SKS03 E1 | 5.20–15.91 | 1.049 | 229.02 | 0.008 |
| SKS03 E2 | 5.06–15.77 | 1.075 | 564.26 | 40.039 |
| SKS03 E3 | 4.69–15.41 | 1.130 | 823.40 | 39.961 |
| SKS03 E4 | 5.41–60.06 | 1.205 | 1185.88 | 41.367 |
| SKS03 E5 | 5.68–67.82 | 1.442 | 1183.93 | 40.000 |

Source: Sinha et al. (2020a, b). Times are from input-motion onset. The pressure peak is a dimensionless multi-channel proxy, not a layer-specific maximum. The preceding recorded interval indicates boundary resolution; it is not a confidence bound. SKS02 EQM1 falls below the operational pressure-response floor. Ten of ten eligible post-shaking events and all 11 shaking events contribute; repeated piles remain aggregated within events.

Table S8. Three-station phase-specific paired profile-error differences

| Source→target | Layout | Phase | n | Median Δ | Event range | − / + |
| --- | --- | --- | --- | --- | --- | --- |
| SKS02→SKS03 | Original | shaking | 5 | −0.106 | −0.151 to −0.074 | 5/0 |
| SKS02→SKS03 | Original | before decay | 5 | −0.115 | −0.161 to −0.071 | 5/0 |
| SKS02→SKS03 | Original | after decay | 5 | −0.098 | −0.154 to −0.066 | 5/0 |
| SKS02→SKS03 | Both ends | shaking | 5 | −0.128 | −0.207 to −0.071 | 5/0 |
| SKS02→SKS03 | Both ends | before decay | 5 | −0.146 | −0.194 to −0.108 | 5/0 |
| SKS02→SKS03 | Both ends | after decay | 5 | −0.116 | −0.172 to −0.089 | 5/0 |
| SKS03→SKS02 | Original | shaking | 6 | +0.023 | −0.044 to +0.057 | 2/4 |
| SKS03→SKS02 | Original | before decay | 5 | +0.008 | −0.044 to +0.057 | 2/3 |
| SKS03→SKS02 | Original | after decay | 5 | +0.012 | −0.042 to +0.045 | 2/3 |
| SKS03→SKS02 | Both ends | shaking | 6 | −0.013 | −0.045 to −0.001 | 6/0 |
| SKS03→SKS02 | Both ends | before decay | 5 | −0.020 | −0.042 to −0.002 | 5/0 |
| SKS03→SKS02 | Both ends | after decay | 5 | −0.004 | −0.033 to +0.005 | 3/2 |

Source: Sinha et al. (2020a, b). Δ is candidate-minus-uniform phase p90 NRMSE after within-earthquake pile aggregation. Negative values favour the candidate. n counts earthquakes; −/+ counts strict event signs, not equivalence classes. Original and boundary-retained layouts were fixed before phase scoring. The small reverse after-decay median includes three lower-error and two higher-error events; it is not evidence of uniformly favourable transfer.

Mirabello evidence qualification

Lusvardi (2020) reports a 17-m-long, 0.25-m-diameter bored, grouted micropile at Mirabello, Italy. The thesis identifies a post-blast acquisition interruption of approximately five minutes, a subsequent strain-baseline adjustment and drift-related exclusions. Strain-derived load is retained only above 6.2 m and for the first 15 min (printed pp. 148–149). The final deeper profile combines CAPWAP side resistance, an assumed toe load and a neutral plane estimated from settlement compatibility (pp. 154–155; Fig. 7-13). Those deeper estimates cannot be treated as independent measured axial-load observations.

The downloaded thesis supplies neither a complete, uninterrupted, synchronised depth-wise axial-load time history nor an independently measured deeper profile for the stated transfer test. It is retained as source qualification and physical context, not added to the measured benchmark. A future dynamic field test requires quality-qualified simultaneous axial-load histories at sufficient depths, gauge geometry and calibration, input motion, pore-pressure records and acquisition/baseline metadata. A synthetic or CAPWAP-derived profile could test software under its assumptions, but could not close this experimental validation gap.

Fig. S1. Forward-sequence evaluation after selecting and freezing each layout from EQM1. Each point is the median across available piles within the later earthquake. Negative candidate-minus-uniform p90 NRMSE favours the frozen layout; the zero line denotes a tie. Source: Data from Sinha et al. (2020a, b).

Fig. S2. Robustness envelopes across four smoothing choices and three NRMSE floors. Points denote the implemented nominal specification; vertical ranges show the minimum and maximum median paired change across all 12 specifications. These are specification envelopes, not confidence intervals. Source: Data from Sinha et al. (2020a, b).

Fig. S3. Co-located physical-regime descriptors and transfer error for the three-added-station comparison. Panels distinguish the centrifuge tests; labels identify earthquakes. Pore-pressure ratio describes the observed test envelope only. No regression significance or causal relation is claimed. Source: Data from Sinha et al. (2020a, b).

Fig. S4. Phase-specific transfer of frozen three-station layouts. Dots show paired earthquake-level differences in phase p90 profile NRMSE; black bars show their medians. Original and boundary-retained layouts share the equal-budget uniform comparator. The decay boundary is a sustained reduction below half the reference pressure-proxy peak, not complete dissipation. Five post-shaking events are available in each direction because SKS02 EQM1 has weak pressure response; shaking uses six SKS02 and five SKS03 events. The 120-snapshot, elapsed-time-smoothed setting is shown; full sensitivities and peak-response errors are provided as CSV files. Source: Sinha et al. (2020a, b).

References for supplementary material

Duffy, K., K. Gavin, M. Korff, and D. de Lange. 2024. “Base resistance of screw displacement piles in sand.” Journal of Geotechnical and Geoenvironmental Engineering 150 (8): 04024070. https://doi.org/10.1061/JGGEFK.GTENG-12340.

Duffy, K., K. G. Gavin, D. A. de Lange, and M. Korff. 2025. “Static load tests on screw injection piles with distributed fibre optic sensors in Delft.” Dataset. 4TU.ResearchData. https://doi.org/10.4121/78720ecb-daf4-4676-b5f4-281e93e4388a.

Lusvardi, C. M. 2020. “Blast-Induced Liquefaction and Downdrag Development on a Micropile Foundation.” M.S. thesis, Brigham Young University, Provo, UT. https://scholarsarchive.byu.edu/etd/8769/.